CW-P-241B LOTO, Noise, and Access Verification

Reference: OSHA 29 CFR 1910.147 lockout/tagout, https://www.osha.gov/laws-regs/regulations/standardnumber/1910/1910.147. Local document owner: Mechanical Maintenance.

# 1. Isolation point checklist

| point_id | energy_or_fluid_source | field_label | normal_position | verified_state | evidence_ref | note |
| --- | --- | --- | --- | --- | --- | --- |
| LOTO-01 | 480 V motor feeder | MCC-CW-241B | closed | locked open | LOTO photo LP-01 | Primary electrical isolation. |
| LOTO-02 | VFD local enable | VFD-CW-241B | enabled | disabled and tagged | LP-02 | Secondary start-prevention control. |
| LOTO-03 | suction header | SV-241B | open | locked closed | LP-03 | Valve chained closed. |
| LOTO-04 | discharge header | DV-241B | open | locked closed | LP-04 | Valve locked and verified no backflow. |
| LOTO-05 | seal flush tapping | FV-241B-FS1 | open | locked closed | LP-05 | Omitted from older 5-point register count. |
| LOTO-06 | casing drain | DR-241B | closed | open and tagged | LP-06 | Drained to closed sump. |
| LOTO-07 | instrument impulse line | IV-FT241B | open | closed and tagged | LP-07 | Added for flowmeter maintenance. |

# 2. Noise survey

| survey_point | distance_m | measured_dba | duration_min | control_note | status |
| --- | --- | --- | --- | --- | --- |
| Pump DE walkway | 1.0 | 89.2 | 12 | Double hearing protection during local inspection | REVIEW |
| Motor NDE walkway | 1.0 | 87.6 | 12 | Hearing protection required | ACCEPTABLE_WITH_PPE |
| Operator log station | 6.0 | 78.4 | 30 | No additional controls | ACCEPTABLE |
| Temporary scaffold landing | 1.5 | 88.8 | 10 | Limit dwell time and post sign | REVIEW |

# 3. Access checks

The coupling guard has 8.5 mm measured clearance at the closest rotating element.

The seal flush valve is accessible only after opening the west guard panel.

Access scaffold is not required for routine vibration readings but is required for seal flush line tracing.

Noise readings are operational exposure context and do not change mechanical geometry, alignment, or flow records.

# 4. Reconciliation notes

The current LOTO checklist has 7 isolation points. The older asset register count of 5 is incomplete because it does not include the seal flush tapping or instrument impulse line.